TFT_eSPI Library Settings

# Anpassungen in User_Setup_Select.h:

Include für User_Setup auskommentieren:

//#include <User_Setup.h> // Default setup is root library folder

Include für ESP32_S3 Setup aktivieren:

#include <User_Setups/Setup70b_ESP32_S3_ILI9341.h> // Setup file for ESP32 S3 with SPI ILI9341

Inhalt von Setup70b_ESP32_S3_ILI9341.h anpassen:

// Setup for the ESP32 S3 with ILI9341 display // Note SPI DMA with ESP32 S3 is not currently supported #define USER_SETUP_ID 70 // See SetupX_Template.h for all options available #define ILI9341_DRIVER // Typical board default pins - change to match your board #define TFT_CS 10 // 10 or 34 (FSPI CS0) #define TFT_MOSI 11 // 11 or 35 (FSPI D) #define TFT_SCLK 12 // 12 or 36 (FSPI CLK) #define TFT_MISO 13 // 13 or 37 (FSPI Q) // Use pins in range 0-31 #define TFT_DC 14 #define TFT_RST 47 #define TFT_BL 35 // LED back-light #define TFT_BACKLIGHT_ON HIGH // HIGH schaltet die Beleuchtung ein // Font settings #define LOAD_GLCD #define LOAD_FONT2 #define LOAD_FONT4 #define LOAD_FONT6 #define LOAD_FONT7 #define LOAD_FONT8 #define LOAD_GFXFF #define SMOOTH_FONT // SPI Hardware settings #define USE_HSPI_PORT #define SPI_FREQUENCY 40000000 // Maximum for ILI9341 #define SPI_READ_FREQUENCY 6000000