USB CDC

The USB CDC (Communications Device Class) allows you to communicate to the device like in a serial interface. This mode can be used on the supported targets to flash and monitor the device in a similar way on devices that uses the external serial interfaces.

To use the USB CDC, you need to configure your device in the Tools menu:

- Enter into Download Mode manually

Similar to the DFU mode, you will need to enter into download mode manually. To enter into the download mode, you need to press and hold BOOT button and press and release the RESET button.

To check if this procedure was done correctly, now you will see the new USB device listed in the available ports. Select this new device in the Port option.

- Configure the USB CDC

For ESP32-S2

- USB CDC On Boot -> Enabled

- Upload Mode -> Internal USB

For ESP32-C3

- USB CDC On Boot -> Enabled

For ESP32-S3

- USB CDC On Boot -> Enabled

- Upload Mode -> UART0 / Hardware CDC

- Flash and Monitor

You can now upload your sketch to the device. After flashing for the first time, you need to manually reset the device.

This procedure enables the flashing and monitoring thought the internal USB and does not requires you to manually enter into the download mode or to do the manual reset after flashing.

To monitor the device, you need to select the USB port and open the Monitor tool selecting the correct baud rate (usually 115200) according to the Serial.begin() defined in your code.